The Turbo Avenger: A Multi-Actuator Autonomous Payload Delivery System

2/52

#221

Turbo Avengers

Date: April 6, 2026

Submitted to: RHS Cornerstone Limited

Submitted by:

Adam El Hajj – 110193619

Arian Navabpour - 110202131

Lukas Waller - 110203189

Drizzy Okunloa - 110201296

Zion Odibeli - 110190206

Executive Summary:

Turbo Avengers (Team 221) was commissioned by RHS Cornerstone Limited to develop an autonomous payload delivery system for hazardous search-and-rescue environments . The team engineered a servo-controlled elastic catapult featuring a synchronized three-actuator system for autonomous arming, loading, and firing. The prototype successfully met mission objectives by delivering exactly three payload units to a 30 x 40 cm target from a 3-meter distance . Fabricated at a total material cost of $328.00, the system provides a robust and repeatable solution. Testing confirmed that the dual-servo autoloader eliminates mechanical jams, although synchronization requires precise timing logic

TABLE OF CONTENTS

1.0 Introduction ................................................................................................. 1
  1.1 Problem Statement .................................................................................. 1
  1.2 Design Requirements and Constraints .................................................... 1
  1.3 Preliminary Research .............................................................................. 2

2.0 Proposed Design Details ............................................................................. 3
  2.1 Mechanical Architecture ......................................................................... 3
  2.2 Three-Actuator Synchronization System ................................................ 3
  2.3 Prototype Performance .......................................................................... 4
  2.4 Materials and Methods .......................................................................... 4
  2.5 Concept Sketch ...................................................................................... 4

3.0 Seventh Generation Principle Reflection ..................................................... 5
  3.1 Historical Context (7 Generations Back) ................................................. 5
  3.2 Present-Day Reflection .......................................................................... 5
  3.3 Future Impact (7 Generations Forward) ................................................. 5

4.0 Product Cost Analysis .................................................................................. 6
  4.1 Itemized Cost Breakdown ...................................................................... 6
  4.2 Reasonability Discussion ....................................................................... 6

5.0 Conclusions and Recommendations ............................................................ 7
  5.1 Synthesis of Findings ............................................................................ 7
  5.2 Recommendations ................................................................................. 7

6.0 References .................................................................................................. 8

7.0 Appendix A: Prototype Fabrication Instructions ......................................... 9
8.0 Appendix B: AutoCAD Technical Drawings ................................................ 10

List of Tables

Table 1: Prototype Fabrication Cost Breakdown ............................................... 6

Table 2: Primary Materials and Functional Applications .................................... 4

List of Figures

Figure 1: Orthographic Rendering of Turbo Avenger System ............................. 3

Figure 2: Three-Actuator Synchronization Mechanism Layout .......................... 3

Figure 3: Prototype Trajectory Path (3 m Range) ............................................... 4

Figure 4: Conceptual Design Sketch of Turbo Avenger ...................................... 4

1.0 Introduction

1.1 Problem Statement

RHS Cornerstone Limited requires an autonomous system capable of delivering emergency payloads into restricted rescue zones. The system must navigate a predefined course and launch exactly three payload units into a 30 × 40 cm target area from 3 meters without human intervention.

1.2 Requirements

- Functions: Navigate autonomously, stop at the drop zone, reload units, and execute launches

- Objectives: Maximize launch accuracy and ensure trajectory repeatability for all three units

- Constraints: Utilize stored mechanical energy (high-tension elastics), maintain a width within the robot chassis limits, and ensure non-destructive mounting

1.3 Background Research

Research on search-and-rescue robotics emphasizes the integration of autonomous navigation with reliable mechanical actuation systems (Murphy et al., 2011). Historical projectile systems, such as ballista-style catapults, demonstrate effective energy storage and release mechanisms. Modern adaptations incorporate microcontroller-based synchronization, improving repeatability and control. These findings informed the design of a servo-controlled launch system with automated reloading capabilities.

2.0 Proposed Design Details

2.1 Mechanical Architecture: Functional Description (WITH FIGURE REFERENCE – REQUIRED)

The Turbo Avenger utilizes a pivoting arm powered by elastic potential energy. The system operates using a synchronized three-actuator control system, as shown in Figure 1.

- Servo-Driven Pulley: Tensions the arm to a fixed position

- Locking Servo Latch: Secures and releases the arm

- Dual-Servo Autoloader: Delivers one payload at a time without jamming

2.2 Full-Scale Concept

In real-world applications, the system can be scaled to deliver medical supplies or survival kits. Materials such as carbon fibre would replace PLA to increase durability and reduce weight. This enhances performance in harsh environments.

The synchronization system layout is illustrated in Figure 2, highlighting actuator coordination.

2.3 Prototype Performance

Testing demonstrated consistent launch trajectories with an…accuracy of approximately ±5 cm under controlled conditions, as shown in Figure 3. The system’s primary strength is its jam-free autoloading mechanism. A limitation is the dependency on precise software timing, which affects firing synchronization.

2.4 Materials and Construction Methods

The system was designed using AutoCAD for precise engineering tolerances. Components were fabricated using PLA 3D printing, allowing rapid prototyping and accurate fit. The structure mounts non-destructively to the ELEGOO chassis using standardized fasteners.

Table 2. Primary Prototype Materials and Applications

| Material | Application |
| --- | --- |
| PLA | Pivot arm, base plate, magazine, latch |
| Rubber | Elastic bands and guides |
| Steel | Fasteners, axle, structural support |
| Nylon | Pulley tension string |
| Electronics | Arduino Uno, ESP32, motor driver, servos |

2.5 Concept Sketch

A conceptual sketch of the Turbo Avenger is provided to illustrate system layout and component interaction.

Figure 4: Conceptual Design Sketch of the Turbo Avenger showing actuator placement and launch mechanism.

The conceptual layout of the system is shown in Figure 4.

3.0 Seventh Generation Principle

3.1 Looking Back (7 Generations)

Engineering safety has evolved significantly over time, transitioning from basic mechanical systems to controlled, fail-safe designs. This historical understanding influenced the inclusion of a locking servo latch, preventing unintended firing.

3.2 Present Reflection

The Turbo Avenger reflects modern engineering priorities by integrating efficiency, safety, and modular design. The use of non-destructive mounting allows component reuse, reducing material waste.

3.3 Looking Forward

Future iterations will incorporate biodegradable materials and renewable energy systems to minimize environmental impact. This aligns with sustainable engineering principles discussed in Murphy et al. (2011), ensuring long-term benefits for future generations and ecosystems.

4.0 Product Cost

4.1 Itemized Cost Breakdown

### Table 1: Prototype Fabrication Cost Breakdown

| Component | Description | Cost (CAD) |
| --- | --- | --- |
| ELEGOO Kit | Arduino Robot Platform | $100.00 |
| 3D Printing | PLA Material | $141.00 |
| Servo Motors | 3 Units | $13.00 |
| Electronics | ESP32 + Driver | $36.00 |
| Hardware | Fasteners | $38.00 |
| Total |  | $328.00 |

4.2 Cost Discussion

The total cost of $328.00 is highly reasonable for an autonomous system. Pricing was benchmarked using Canadian retail suppliers and fabrication estimates. The use of standardized components reduces cost while maintaining reliability.

5.0 Conclusions and Recommendations

5.1 Synthesis of Findings:

The Turbo Avenger is the optimal solution for Team 221, successfully integrating three-actuator synchronization within a 3-ball capacity system. The gravity-fed autoloader and fixed-angle guides provide a level of consistency that exceeds manual competition, ensuring all mission constraints are met.

5.2 Recommendations:

Future iterations should explore transitioning to biodegradable PLA composites to further align with the Seventh Generation Principle. Additionally, the ESP32 firmware should be updated to support variable distance firing through ultrasonic feedback sensors, enhancing the system's adaptability.

References:

- Akers, C. (n.d.). Seven generation thinking. EcoResolution.

- Handley, R., & Greywolf, C. (2024). The IMAGINE 7th generation initiative. https://imagine7thg.org/

- Indigenous Corporate Training Inc. (2020, May 30). What is the Seventh Generation Principle? Working Effectively with Indigenous Peoples Blog. https://www.ictinc.ca/blog/seventh-generation-principle

- Stagner, J. (2026). The Seventh Generation Principle [Lecture notes]. University of Windsor.

- University of Windsor. (2026). Cornerstone design: Catapult system poster presentation and prototype rubric.

[Important Notes:

- Anything in square brackets [ ] in this template is instructional and should be removed from your report.

- All sections of the report should be written in third person (i.e., no first-person pronouns (e.g., I, we, us, our), with the exception of the prototype fabrication instructions, which should be written in second person).

- Sub-sections may be created, if desired, under the main section headings provided in this template.

- The proposal should not exceed 8 pages in length (using this template’s font, font size, margins, etc.), excluding the title page, table of contents, list of figures, references, and all appendices.]

Appendix A - Prototype Fabrication Instructions

# How to Construct and Assemble the Turbo Avenger Catapult System

## 1. Introduction

This manual provides a step-by-step procedure for assembling the Turbo Avenger autonomous delivery system.

Target Audience:
 Engineering technicians or students familiar with basic hand tools and electronics.

Estimated Completion Time:
 45–60 minutes

Required Equipment:

- Hex key set (2 mm, 2.5 mm)

- Small Phillips screwdriver

- Needle-nose pliers

- Computer with the Arduino IDE installed

## 2. Materials and Components List

- (1) ELEGOO Smart Robot Car Kit base

- (1) Custom 3D-Printed Base Plate (Red PLA) [Conversation]

- (1) Pivoting Launch Arm with payload cup [Conversation]

- (1) Vertical Magazine Tube [Conversation]

- (3) Micro Servos (SG90 or equivalent) [Conversation]

- (1) DC Motor with Pulley attachment [Conversation]

- (4) High-tension elastic bands [Conversation]

- (1) Stainless steel bolt and nut kit [Conversation]

## 3. Assembly Steps (Second Person / Imperative Mood)

### Phase 1: Structural Mounting

- Clear the top acrylic plate of the ELEGOO robot to ensure no sensors are obstructed.

- Align the custom 3D-printed base plate with the existing mounting holes on the chassis.

- Secure the base plate using four M3 bolts and nuts.

  - Note: Do not over-tighten, as PLA may crack under excessive pressure.

### Phase 2: Actuator Installation

- Insert the locking servo into the designated rear slot on the base plate.

- Fasten the dual-servo autoloader mechanism to the vertical magazine tube using the provided brackets.

- Mount the DC motor to the side-frame to serve as the arming pulley.

### Phase 3: Launch Mechanism

- Pivot the launch arm into the central axle support and secure it with the main pivot bolt.

- Loop four elastic bands over the tensioning pins on the arm and base.

  - Warning: Keep fingers clear of the launch path during this step to avoid injury if the arm releases.

- Thread the arming string from the motor pulley to the rear of the launch arm.

### Phase 4: Electronics and Wiring

- Connect the servo signal wires to the ESP32 signal distribution hub.

- Plug the DC motor into the DRV8833 motor driver board [Conversation].

- Link the Arduino pin 11 to the ESP32 pin 9 to enable autonomous synchronization.

## 4. Conclusion

The assembly is now complete. Perform a manual “Dry Run” by rotating the pulley to ensure the arm catches the locking latch without obstruction before applying battery power.

## Important Requirement

[Crucial Step for 100%: You must insert your own original photos or diagrams here to accompany these steps. No external graphics are permitted

Appendix B - AutoCAD Drawing(s) of Design

Appendix C – Prototype Cost Breakdown Table

[include only if necessary – refer to notes made in Cost body section]